### 1.1 Hardware Requirements

The following hardware is required to reproduce and run the CapsNet accelerator on the ZCU102 development board.

| Hardware | Description / Purpose |
| --- | --- |
| ZCU102 Development Board | AMD/Xilinx Zynq UltraScale+ MPSoC development board used to implement and evaluate the CapsNet accelerator. |
| USB-UART Cable | Connects the ZCU102 board to the host laptop for serial communication, boot monitoring and debugging. |
| Power Cord and Power Supply | Provides power to the ZCU102 development board. |
| Ethernet Cable | Connects the ZCU102 to the host laptop for network communication and file transfer. |
| Host Laptop / Workstation | An x86-64 computer capable of running Ubuntu 20.04 LTS and the Xilinx development toolchain. |

Additional requirement: A compatible SD card is required to boot the ZCU102 using the prepared PetaLinux image.

1.2 Software Setup and Walk-Up Lab

This section describes the software environment required to reproduce the CapsNet accelerator project. The development environment is based on Ubuntu 20.04 LTS and the AMD/Xilinx 2022.1 toolchain.

1.2.1 Operating System

The host computer should run Ubuntu 20.04 LTS (64-bit), which is officially supported by the Xilinx 2022.1 development tools.

If Ubuntu is not already installed, follow the official installation guide:

https://ubuntu.com/tutorials/install-ubuntu-desktop

After installation, open a terminal and verify the operating system:

lsb_release -a uname -m

The expected Ubuntu release is 20.04.x, and the architecture should be x86_64.

1.2.2 Walk-Up Lab — Linux Environment Preparation

Before installing the FPGA development tools, update the operating system and install essential development utilities.

sudo apt update sudo apt upgrade -y

Install commonly used development packages:

sudo apt install -y \ build-essential git wget curl \ unzip zip make gcc g++ \ python3 python3-pip

Verify the installation:

gcc --version g++ --version python3 --version git --version

1.2.3 Required Development Software

| Software | Version | Purpose |
| --- | --- | --- |
| Ubuntu | 20.04 LTS | Host operating system |
| Vivado | 2022.1 | FPGA hardware design and implementation |
| Vitis | 2022.1 | Hardware linking and system compilation |
| Vitis HLS | 2022.1 | C/C++ to FPGA accelerator compilation |
| Vitis AI | 2.5 | Neural network quantisation and compilation. |
| PetaLinux / SDK | 2022.1 | Embedded Linux and host application development |
| XRT | Compatible with 2022.1 platform | FPGA accelerator runtime |
| Python | Project-compatible version | Model preprocessing and analysis |

Detailed installation and configuration procedures for these tools are provided in the .

1.2.4 Download Project Source Code

Clone the project repository:

git clone https://github.com/wolfuoa/P4P-CapsuleNetworkAccelerator.git cd P4P-CapsuleNetworkAccelerator

## 1.3 Xilinx Toolchain Installation and Configuration

The CapsNet accelerator was developed using the AMD/Xilinx 2022.1 toolchain, including Vivado, Vitis and Vitis HLS. These tools are required to synthesise the custom hardware accelerators, integrate them with the DPU and generate the FPGA configuration files.

### 1.3.1 Download and Install Xilinx Development Tools

- Visit the AMD/Xilinx Downloads page.

- Select 2022.1 from the archived releases.

- Download the Xilinx Unified Installer for Linux.

- Run the installer and select the following tools:

  - Vivado Design Suite 2022.1

  - Vitis 2022.1

  - Vitis HLS 2022.1

  - Zynq UltraScale+ MPSoC device support

- Complete the installation using an appropriate directory, such as /tools/Xilinx/.

For a downloaded Linux installer, follow the instructions provided with the installer. For example, if using the extracted full installation image, execute:

cd <extracted_installer_directory>

./xsetup

### 1.3.2 Configure the Xilinx Environment

After installation, initialise the Xilinx toolchain in a new terminal:

source /tools/Xilinx/Vitis/2022.1/settings64.sh

source /tools/Xilinx/Vivado/2022.1/settings64.sh

Replace /tools/Xilinx/ with the actual installation directory if different.

Verify that the required tools are available:

vivado -version

v++ --version

which vitis_hls

The reported versions should correspond to the 2022.1 release.

These environment setup commands must be executed in each new terminal session before building the FPGA accelerators.

### 1.3.3 Install the ZCU102 Base Platform

The project uses the following embedded platform:

xilinx_zcu102_base_202210_1

Download the ZCU102 base platform and its associated common image from the AMD Vitis Embedded Platforms page.

Extract the platform into a suitable directory, for example:

mkdir -p ~/xilinx/platforms cd ~/xilinx/platforms unzip <ZCU102_platform_archive>.zip

Configure the platform repository path:

export PLATFORM_REPO_PATHS=$HOME/xilinx/platforms

Verify that the platform description is present:

find "$PLATFORM_REPO_PATHS" -name "*.xpfm"

The expected platform file is:

xilinx_zcu102_base_202210_1.xpfm

This platform is used by the Vitis linker when integrating the DPU and custom accelerators into the FPGA system.

Note: The corresponding Zynq UltraScale+ MPSoC common image is also required for the embedded Linux deployment workflow. Its root filesystem, kernel image and cross-compilation sysroot must be configured before generating the final SD card image.

### 1.3.4 Installation Verification

Before proceeding, confirm that:

- Vivado 2022.1 launches successfully.

- Vitis 2022.1 and Vitis HLS 2022.1 are available.

- The ZCU102 base platform has been extracted correctly.

- The .xpfm platform description can be located.

- The Xilinx environment variables are configured.

Detailed instructions for compiling the HLS accelerators and generating the final .xclbin are provided in the subsequent build sections.

## 1.4 Vitis AI Installation

Vitis AI is used to quantise and compile the convolutional layers of the CapsNet model for execution on the DPUCZDX8G accelerator.

The project uses Vitis AI 2.5, including its TensorFlow 2 quantisation and compilation tools.

### 1.4.1 Install Docker

Vitis AI provides a Docker-based development environment containing the required machine learning frameworks, quantisation tools and compilers.

Install Docker on Ubuntu 20.04:

sudo apt update

sudo apt install -y docker.io

Enable and start Docker:

sudo systemctl enable --now docker

Add the current user to the Docker group:

sudo usermod -aG docker $USER

Log out and log back in for the group membership change to take effect.

Verify Docker:

docker --version

docker run --rm hello-world

The second command should run successfully without Docker permission errors.

### 1.4.2 Download Vitis AI 2.5

Clone the official Vitis AI repository and select the required release:

git clone https://github.com/Xilinx/Vitis-AI.git

cd Vitis-AI

git checkout v2.5

Verify the selected release:

git describe --tags --always

The repository should now correspond to Vitis AI 2.5.

### 1.4.3 Prepare the Vitis AI Docker Environment

Vitis AI 2.5 provides a containerised development environment for model quantisation and compilation.

The historical Vitis AI 2.5 Docker image should be used rather than the latest Vitis AI release.

Required configuration:

- Vitis AI release: 2.5

- Framework: TensorFlow 2

- Target accelerator: DPUCZDX8G

- Docker image/tag: [TO VERIFY — insert the exact image used by the project]

Once the appropriate Vitis AI 2.5 image has been identified, start the environment using the Docker launch script provided by the checked-out repository:

cd ~/Vitis-AI

./docker_run.sh <verified_vitis_ai_2.5_image>

Within the container, activate the TensorFlow 2 environment if required by the selected image, and verify that the quantisation and compilation tools are available.

Important: The exact historical Docker image and environment activation commands must be recorded from the working project environment. A newer Vitis AI container must not be substituted without compatibility testing.

### 1.4.4 Verify the Vitis AI Environment

Inside the Vitis AI development environment, verify the TensorFlow installation:

python3 -c "import tensorflow as tf; print(tf.__version__)"

Check whether the TensorFlow 2 quantisation module is available:

python3 -c "import tensorflow_model_optimization"

Verify the Vitis AI compiler:

which vai_c_tensorflow2

Successful verification indicates that the required Python environment and compiler are accessible. The exact installed versions should be saved as part of the Compendium environment record.

### 1.4.5 CapsNet Model Preparation Workflow

After installation, the CapsNet model is prepared for DPU execution using the following workflow:

CapsNet.h5

|

v

create_partial_model.py

|

v

partial_model.h5

|

v

Vitis AI TensorFlow 2 Quantiser

|

v

quantized_partial_model.h5

|

v

Vitis AI TensorFlow 2 Compiler

|

v

partialCaps.xmodel

The convolutional portion of the network is processed using Vitis AI, while the remaining capsule operations are implemented using custom HLS accelerators.

The precise quantisation settings, calibration dataset, compiler configuration and DPU architecture description are documented in the model compilation section.

### 1.4.6 Installation Verification Checklist

Before proceeding to the model compilation workflow, confirm that:

- Docker is installed and functioning.

- The Vitis AI repository is checked out at release 2.5.

- The correct TensorFlow 2 development environment is accessible.

- TensorFlow imports successfully.

- The Vitis AI quantisation tools are available.

- The vai_c_tensorflow2 compiler is accessible.

- The DPU architecture configuration matches the FPGA implementation.